King Fahd University of Petroleum and Minerals

## Computer Engineering Department

# COE 443: High-Speed Networks

Fall 2001 (011)

Student ID#:			Student Name:

Q.1 [10] ATM is gaining a lot of interest for deployment in LAN technologies. What are the requirements of these LAN technologies in order to use ATM?

Q.2 [10] Messages arrive at a switching node for a particular outgoing communication line in a Poisson manner with mean arrival rate of 180 messages per hour. Message length is distributed exponentially with mean length of 14400 octets. Line speed is 9600 bps.

- What is the mean waiting time in the switching node?

b.  How many messages will be waiting in the switching node for transmission on average?