COE 205

Introduction to Computer Organization and Assembly Language

Input/OUTPUT instructions

Input/Output Instructions  	2

Objectives:	2

Introduction	2

Accessing I/O Ports	3

Register I/O Instructions	3

Block I/O Instructions	4

Input/Output Instructions

Objectives:

- To describe various ways of interacting with input/output devices

- To describe register I/O instructions: IN and OUT

- To describe Block I/O instructions: INS and OUTS

Introduction

There are three ways of interacting with input/output devices by an application program. One way is through the use of the DOS interrupt service routines. Another way is through the use of the BIOS interrupt service routines. The BIOS and DOS interrupt service routines provide many services that are needed by an application program like reading from the keyboard, displaying to the screen, interacting with the mouse, interacting with the serial and parallel ports.

While the DOS interrupt service routines are standard, the BIOS service routines are not. The specifications of the routines are standard, however their implementation differ from a manufacturer to another. This is because there is no standard for assigning addresses to input/output devices.

It is also possible for an application program to interact directly with input/output devices. This is necessary when there is no such support for the device available either from DOS or BIOS, or when we want a nonstandard access. The various ways of interacting with I/O devices is illustrated below.

Fig. m20010.1 Various ways of interacting with I/O devices

In order to communicate with input/output devices directly, we need to know their assigned addresses and also the protocol of communication with them. Input/Output devices are assigned addresses in the range of the first 64K byte in the same memory address map. In order to distinguish between the addresses used by the I/O devices and those used by the memory, I/O addresses are called ports. Pentium uses a pin to distinguish whether the address is for memory access or I/O device access. The address space can be used for 8-bit, 16-bit, or 32-bit I/O ports.

Accessing I/O Ports

Pentium provides two types of instructions for accessing I/O ports:

- Register I/O instructions  &

- Block I/O instructions

Register I/O instructions are used to transfer data between the accumulator (Al, AX, or EAX) and an I/O port. Block I/O instructions are used for block transfer of data between memory and I/O ports.

Register I/O Instructions

Pentium provides two register I/O instructions: IN and OUT. The IN instruction is used to read data from an I/O port, and the OUT instruction is used to write data to an I/O port. A port address can be any value in the range 0 to FFFFH. The first 256 ports (i.e. ports with address in the range 0 to FFH) are directly addressable -i.e., the port address is given as part of the instruction- by IN and OUT instructions. Port addresses greater than 255 have to be stored in the DX register. Both IN/OUT instructions can be used to read/write 8-, 16-, or 32-bit data. The syntax and semantics of the IN and OUT instructions are illustrated in the table below:

| Instruction | Description |
| --- | --- |
| IN AL, port | AL<-[port] |
| IN AL, DX | AL<-[DX] |
| IN AX, port | AX<-[port+1:port] |
| IN AX, DX | AX<-[DX+1:DX] |
| IN EAX, port | EAX<-[port+3:port] |
| IN EAX, DX | AX<-[DX+3:DX] |
| OUT port, AL | [port]<-AL |
| OUT DX, AL | DX]<-AL |
| OUT port, AX | [port+1:port]<-AX |
| OUT DX, AX | [DX+1:DX]<-AX |
| OUT port, EAX | [port+3:port]<-AX |
| OUT DX, EAX | [DX+3:DX]<-AX |

Note that the port address is the source in the IN instruction while it is the destination in the OUT instruction signifying the direction of the data movement. The accumulator operand, (AL, AX, or EAX), in the IN/OUT instructions determines whether a byte, word, or double word it to be Read/Written from/to the specified port.

For example, the instruction IN AX, 20 will read a word starting from port 20 and store it in AX. So, the AL register will get a byte from port 20 while the AH register will get a byte from port 21. Similarly, the instruction OUT 20, EAX will output a double word from register EAX to the ports [23:20].

Block I/O Instructions

Pentium provides the following block I/O instructions that can be used to move blocks of data between I/O ports and memory:

INS:  INSB,  INSW,  INSD

OUTS: OUTSB. OUTSW, OUTSD

These instructions are similar to string instructions. Like the move string instructions, the repeat prefix REP can be used with these instructions. The port address has to be specified in the DX register.

For the INS instructions, a byte, word, or double word is input from the specified port in DX and stored in the corresponding locations pointed by ES:[DI]. Then, the DI index is either incremented (DF=0) or decremented (DF=1) by 1 (INSB), 2 (INSW), or 4 (INSD).

For the OUTS instructions, a byte, word, or double word is output from the locations pointed by DS:[SI] to the corresponding specified port in DX. Then, the SI index is either incremented (DF=0) or decremented (DF=1) by 1 (OUTSB), 2 (OUTSW), or 4 (OUTSD).

The syntax and semantics of the INS and OUTS instructions are illustrated in the table below:

| Instruction | Description |
| --- | --- |
| INSB | ES:[DI]<-[DX] ; IF (DF=0) DI=DI+1 ELSE DI=DI-1 |
| INSW | ES:[DI+1:DI]<-[DX+1:DX] ; IF (DF=0) DI=DI+2 ELSE DI=DI-2 |
| INSD | ES:[DI+3:DI]<-[DX+3:DX] ; IF (DF=0) DI=DI+4 ELSE DI=DI-4 |
| OUTSB | [DX]<-DS:[SI] ; IF (DF=0) SI=SI+1 ELSE SI=SI-1 |
| OUTSW | [DX+1:DX]<-DS:[SI+1:SI] ; IF (DF=0) SI=SI+2 ELSE SI=SI-2 |
| OUTSD | [DX+3:DX]<-DS:[SI+3:SI] ; IF (DF=0) SI=SI+4 ELSE SI=SI-4 |