COE 205

Introduction to Computer Organization and Assembly Language

Interrupts

Objectives:	3

Introduction	3

Interrupts vs. Procedures	3

A Taxonomy of Interrupts  	3

Software Interrupts	4

Hardware Interrupts	4

Interrupt Flag (IF)	4

Identifying Hardware Interrupt Types	5

Handling Interrupts from Several I/O Devices	5

Processor Interrupts or Exceptions	5

Interrupt Processing  	5

BIOS Interrupts 	7

Interrupt Types 0-7	7

Types 8h-Fh	7

Interrupt Types 10h-1Fh	8

Interrupt 16h -- Keyboard I/O Example	8

Interrupt 17h -- Printer I/O	9

Interrupts 1Dh-1Fh	10

DOS Interrupts	10

Interrupt 21h -- Function Request	10

Extended Keyboard Keys	12

A Time Display Program  	12

User Interrupt Procedures	13

Set Interrupt Vector	13

Cursor Control	14

Interrupt Procedure	15

File Processing  	16

File Handle	16

File Errors	17

Opening a New File	17

Opening an Existing File	18

Closing a File	18

Reading from a File	18

Writing to a File	19

Objectives:

- To describe the interrupt mechanism of Pentium

- To describe hardware, software, and processor interrupts

- To describe Interrupt processing

- To briefly describe BIOS interrupts

- To briefly describe DOS interrupts

- To illustrate the use of interrupts in a time display program

- To describe how to write an interrupt service routine

- To briefly describe file processing using INT 21h

Introduction

Interrupt is a mechanism by which a program's flow of control can be altered. When an interrupt occurs, the CPU suspends its execution of the current program, and transfers control to an Interrupt Service Routine (ISR), also called a Handler, that will provide the requested service by the interrupt. When the ISR is completed, the original program resumes execution as if it were not interrupted.

Interrupts vs. Procedures

Although the behavior of interrupts is analogous to procedures, there are some basic differences that make interrupts almost indispensable. These differences are highlighted below:

- Interrupts can be initiated by both software and hardware; procedures can be initiated only by software.

- Interrupt mechanism provides an efficient way to handle unanticipated events. For example, if the program goes into an infinite loop, ctrl-break could cause an interrupt to suspend the program execution.

- Interrupt Service Routines are memory resident while procedures are loaded with application programs.

- Interrupts are identified by numbers while procedures are identified by names.

A Taxonomy of Interrupts

| - There are three main types of interrupts: | A taxonomy of Pentium interrupts is shown below: |
| --- | --- |
| - Software interrupts - Hardware interrupts - Processor interrupts or Exceptions |  |

Software Interrupts

Software interrupts are initiated by executing the interrupt instruction INT in a program. Software interrupts are mainly used in accessing I/O devices such as the keyboard, printer, screen, disk drive, etc. For example, INT 21H instruction is used to read a character or a string from the keyboard or display a character or a string on the screen. Software interrupts can be classified into system-defined or user-defined. System-defined software interrupts are those whose interrupt service routines are supported by BIOS and DOS. User-defined interrupts are those whose interrupt service routines are provided by the user. The format of the interrupt instruction is:

INT interrupt-type

where interrupt-type is an integer in the range 0 through 255. Thus, there are 256 different interrupt types. This is a sufficiently large number, as each interrupt type can be parameterized to provide several services. For example, there are more than 80 different services (called functions) provided by DOS through INT 21H. The following table shows which of the 256 interrupt types interrupts are BIOS interrupts, DOS interrupts, or user interrupts.

| Interrupt Type | Allocation |
| --- | --- |
| 0-1Fh | BIOS Interrupts |
| 20h-3Fh | DOS Interrupts |
| 40h-7Fh | Reserved |
| 80h-F0h | ROM Basic |
| F1h-FFh | User Defined |

In addition to the INT instruction, there is only one additional conditional software interrupt instruction, INTO, interrupt on overflow, which invokes INT 4 when the overflow flag is set. Note that the interrupt instructions have no effect on the flags.

Hardware Interrupts

Hardware interrupts are generated by hardware devices to get the attention of the CPU. For example, when a key is pressed, the keyboard generates an interrupt causing the CPU to suspend its present activity and execute the keyboard interrupt service routine to process the key. Hardware interrupts can be either maskable or non-maskable. Maskable interrupts are initiated through INTR pin while non-maskable interrupts are initiated through the NMI pin. The non-maskable interrupts are serviced by the CPU immediately after completing the execution of the current instruction. However, maskable interrupts can be delayed until execution reaches a convenient point. One example of non-maskable interrupts is the RAM parity error indicating memory malfunction.

Interrupt Flag (IF)

The Interrupt Flag (IF) controls whether maskable interrupts are delayed or not. When IF=1, the maskable interrupts will be serviced by the CPU. When IF=0 the maskable interrupts will be delayed until the IF becomes 1. The instruction STI / CLI is used to set /clear the interrupt flag.

Identifying Hardware Interrupt Types

In response to a hardware interrupt request on the INTR pin, the CPU initiates an interrupt acknowledge sequence. As part of this sequence, the CPU sends out an interrupt acknowledge (INTA) signal, and the interrupting device places the interrupt type number on the data bus.

Handling Interrupts from Several I/O Devices

Most hardware interrupts are maskable interrupts. Thus, all interrupts requested from external devices are initiated through the INTR pin. When more than one device interrupts, we have to have a mechanism to prioritize these interrupts and forward only one interrupt request at a time to the CPU while keeping the other interrupt requests pending for their turn. This mechanism can be implemented by using a special chip - the Intel 8259 Programmable Interrupt Controller.

Processor Interrupts or Exceptions

Exceptions are processor interrupts to handle instruction faults. An example of an exception is the divide error fault, which is generated when the quotient can not fit in the quotient register. Exceptions are classified into three types depending on the way they are reported and whether or not the instruction that interrupted is restarted:  Faults, Traps & Aborts

Faults and traps are reported at instructions boundaries. When a fault is reported, the system state is restored to the state before the instruction that caused the interrupt so that the instruction can be restarted. Examples of faults are the divide error fault and the segment-not-present fault. The segment-not-present fault is caused by a reference to data in a segment that is not in memory. Then, the exception handler must load the missing segment from the hard disk and resume program execution starting with the instruction that caused the exception.

On the other hand, when a trap is reported, the system state is restored to the state after the instruction that caused the interrupt. An example of a trap is the overflow exception (INT 4). Aborts are exceptions that report severe errors. Examples include hardware errors and inconsistent values in system tables.

Interrupt Processing

When the CPU is started, the BIOS and DOS ISRs are loaded into memory and they stay memory resident. In order to have flexibility in storing these ISRs in memory in any location, their addresses are stored in an interrupt descriptor table (IDT) or interrupt vector table (IVT).

In protected mode the IDT can be stored at any location and its address is stored at the 48-bit register IDTR. In this case, the address of an ISR requires 8 bytes. So, the size of the IDT is 2048 bytes. However, in real mode the IDT is stored at base address 0. In this case, the address of an ISR requires 4 bytes only, 2 bytes for the IP offset and 2 bytes for the CS. Since each entry in the IVT requires 4 bytes, the interrupt type is multiplied by 4 to get the corresponding ISR pointer in the table.

For example, the ISR pointer for INT 0 is 0000h, the pointer for INT 1 is 0004h, the pointer for INT 2 is 0008h, and the pointer for INT 21h is 0084h. For INT 21h, the IP register will be loaded with the 16-bit from addresses [0085] and [0084] and the CS register will be loaded with the 16-bit from addresses [0087] and [0086].

| This figure illustrates the input vector layout in memory for the real mode. When an interrupt occurs, the following action is taken by the CPU: - Push flags register onto the stack, - Clear interrupt and trap flags, - Push CS onto the stack, - Push IP onto the stack, - Load CS with the 16-bit at memory address [interrupt-type*4+2], - Load IP with the 16-bit at memory address [interrupt-type*4]. |  |
| --- | --- |

Just like procedures, ISRs should end with a return statement to send control back to the interrupted program. The interrupt return instruction, IRET, is used for this purpose. When the IRET instruction is executed, the CPU performs the following steps:

- Pop the 16-bit from the top of the stack into the IP register,

- Pop the 16-bit from the top of the stack into the CS register,

- Pop the 16-bit from the top of the stack into the flags register.

BIOS Interrupts

Interrupt types 0 to 1Fh are known as BIOS interrupts: most ISRs are BIOS routines residing in the ROM.

Interrupt Types 0-7

Interrupt types 0-7 are reserved by Intel: types 0-5 are predefined, 6 and 7 are not yet used.

| 00h | Divide Overflow | Generated when a DIV or IDIV operation produces an overflow. This occurs when the quotient can not fit in the destination register. The interrupt 0 routine displays the message "DIVIDE OVERFLOW" and returns control to DOS. |
| --- | --- | --- |
| 01h | Single Step | Single-stepping is a useful debugging tool to observe the behavior of a program instruction by instruction. This interrupt is generated when the Trap Flag (TF) is set. The ISR can be used to display relevant information about the state of the program (for example, the contents of all registers). Single-stepping is stopped by clearing TF. There are dedicated instructions for setting and clearing the TF. |
| 02h | Non-Maskable Interrupt | Interrupt 2 is the hardware interrupt that cannot be masked out by clearing the TF. The IBM PC uses this interrupt to signal memory and I/O parity errors that indicate bad chips. |
| 03h | Breakpoint | The only 1-byte interrupt instruction (opcode: CCh); other interrupt instructions are 2-byte instructions. Inserting a breakpoint in a program involves replacing the program code byte by CCh while saving the code byte for later restoration to remove the breakpoint. Breakpoints are used by program debuggers. |
| 04h | Overflow | Generated by the instruction INTO (interrupt if overflow) when the OF is set. Programmers may write their own routines to handle unexpected overflows. Executing the instruction INT 4 will invoke the ISR for this interrupt unconditionally while INTO invokes it according to OF. The overflow condition is usually detected and processed using the instructions JO and JNO not INTO. |
| 05h | Print Screen | The BIOS interrupt 5 routine sends the video screen information to the printer. An INT 5 instruction is generated by the keyboard interrupt routine (interrupt 9) when the PrtSC (print screen) key is pressed. |

Types 8h-Fh

The 8086 has only one pin, INTR pin, for maskable hardware interrupt signals. To allow more devices to interrupt the 8086, the Intel 8259 Programmable Interrupt Controller chip is used. It can interface up to 8 devices. Interrupt types 8h-Fh are generated by hardware devices connected to the 8259. The older PCs uses only 8, 9, and Eh.

| 08h | Timer | The IBM PC contains a timer circuit that generates an interrupt once every 54.92 milliseconds (about 18.2 times per second). The BIOS interrupt 8 routine services the timer circuit. It uses the timer signals (ticks) to keep track of the time of the day. |
| --- | --- | --- |
| 09h | Keyboard | Generated by the keyboard each time a key is pressed or released. It reads a scan code and stores it in the keyboard buffer. In addition, it also identifies special key combinations such ctrl-break. The keyboard buffer has the capacity to store up to 15 keys. When the buffer is full, pressing a key causes the BIOS to beep, indicating that the key stroke is lost. The keyboard controller supplies the key identity by means of a scan code. The scan code of a key is simply an identification number given to the key based on its location in the keyboard. The scan code has no relation to the ASCII code. The interrupt 9 routine receives the scan code and generates the equivalent ASCII code, if there is one. Both the scan code and the ASCII code are placed in the keyboard buffer. |
| 0EH | Diskette Error | The BIOS interrupt Eh handles diskette errors. |

Interrupt Types 10h-1Fh

Interrupt 10h-1Fh can be called by application programs to perform various I/O operations and status checking. Associated with each I/O device there is a device controller or I/O controller that acts as a hardware interface between the processor and the I/O device. The device controller performs many of the low-level tasks specific to the I/O device. This allows the CPU to interact with the device at higher level. For each device controller, there is a software interface that provides a clean interface to access the device. This software interface is called the device driver.

| 10H | Video | The BIOS interrupt 10h routine is the video driver. |
| --- | --- | --- |
| 11H | Equipment Check | The BIOS interrupt 11h routine returns the equipment configuration of the particular PC. The return code is placed in register AX. |
| 12H | Memory Size | This interrupt returns (in KB) the amount of conventional memory a computer has in register AX. Conventional memory refers to memory circuits with addresses below 640KB. |
| 13H | Disk I/O | The BIOS interrupt 13h routine is the disk driver. It allows application programs to do disk I/O. |
| 14H | Communications | The BIOS interrupt 14h routine is the communications driver that interacts with the serial ports. |
| 15H | Cassette | This interrupt was used by the original PC for cassette interface. |
| 16H | Keyboard I/O | The BIOS interrupt 16h routine is the keyboard driver. |
| 17H | Printer I/O | The BIOS interrupt 17h routine is the printer driver. |
| 18H | BASIC | The BIOS interrupt 18h routine transfers control to ROM BASIC. |
| 19H | Bootstrap | The BIOS interrupt 19h routine reboots the system. |
| 1AH | Time of Day | The BIOS interrupt 1Ah routine allows a program to get and set the timer tick count. |
| 1BH | Ctrl Break | This interrupt is called by the INT 9 routine when the Ctrl-break key is pressed. The ISR contains only an IRET instruction. Users may write their own routine to handle the Ctrl-break key. |
| 1CH | Timer Tick | Interrupt 1Ch is called by the INT 8 routine each time the timer circuit interrupts. The ISR contains only an IRET instruction. Users may write their own service |

Interrupt 16h -- Keyboard I/O Example

We list here some examples. To read a character from the keyboard, function 0 in AH is used as follows:

Function 00H -- Read a character from the keyboard Input: AH=00H Returns: if AL<>0 then AL = ASCII code of the key entered AH = Scan code of the key entered if AL = 0 AH = Scan code of the extended key entered

This BIOS function can be used to read a character from the keyboard. If the keyboard buffer is empty, it waits for a character to be entered. The value returned in AL determines if the key represents an ASCII character or an extended key character. In both cases, the scan code is placed in the AH register and the ASCII and scan codes are removed from the keyboard buffer. To check if keyboard buffer is empty or not, we can use function 1 as follows:

Function 01H -- Check keyboard buffer Input: AH=01H Returns: ZF= 1 if the keyboard buffer is empty ZF= 0 if there is at least one character available. In this case, the ASCII and scan codes are placed in the AL and AH registers as in function 00. The codes, however, are not removed from the buffer.

Function 2 can be used to check keyboard status with regard to shift and toggle keys.

Function 02H – Check keyboard status Input: AH=02H Returns: AL= status of the shift and toggle keys

The following table indicates the bit assignment for shift and toggle keys. A bit with a value of 1 indicates the presence of a condition.

| Bits in AL | Key Assignment |
| --- | --- |
| 7 | Ins lock switch is on |
| 6 | Caps lock switch is on |
| 5 | Num lock switch is on |
| 4 | Scroll lock switch is on |
| 3 | Alt key depressed |
| 2 | Ctrl key depressed |
| 1 | Left shift key depressed |
| 0 | Right shift key depressed |

Interrupt 17h -- Printer I/O

The BIOS interrupt 17h routine is the printer driver. The routine supports three functions: 0-2.

- Function 0 writes a character to a printer; input values are AH=0, AL=character to be printed, DX=printer number (0=LPT1, 1=LPT2, 3=LPT3).

- Function 1 initializes a printer port; input values are AH=1, DX=printer number.

- Function 2 gets printer status; input values are AH=2, DX= printer number.

For all functions the status is returned in AH. The following table shows the meaning of the bits returned in AH:

| Bits in AH | Meaning |
| --- | --- |
| 7 | = 1 printer not busy |
| 6 | = 1 print acknowledge |
| 5 | = 1 out of paper |
| 4 | = 1 printer online |
| 3 | = 1 I/O error |
| 2 | = 1 not used |
| 1 | = 1 not used |
| 0 | = 1 printer timed-out |

Next, we show an example for printing character 0 on the printer. Because printers contain buffers for data, the 0 will not be printed until a carriage return or line feed character is sent.

Example: Print a 0 on the printer.

MOV AH, 0 ; function 0, print character MOV AL, '0' ; character to be printed in AL MOV DX, 0 ; Printer 0 (LPT1) INT 17H ; AH contains return code MOV AH, 0 ; function 0, print character MOV AL, 0AH ; line feed INT 17H

Interrupts 1Dh-1Fh

These interrupt vectors point to data instead of instructions. The interrupt 1Dh, 1Eh, and 1Fh point to video initialization parameters, diskette parameters, and video graphics characters, respectively.

DOS Interrupts

The interrupt types 20h-3Fh are serviced by DOS routines that provide high-level service to hardware as well as system resources such as files and directories. The most useful is INT 21H, which provides many functions for doing keyboard, video, and file operations.

| 20h | Program Terminate | Interrupt 20h can be used to return control to DOS. Because CS must be set to the program segment before using INT 20h, it is more convenient to use INT 21H, function 4Ch to exit. |
| --- | --- | --- |
| 21h | Function Request | This routine provides over 80 functions that maybe classified as character I/O, file access, memory management, disk access, networking, and miscellaneous. |
| 22h 26h |  | Ctrl -Break, critical errors, and direct disk access. |
| 27h | Terminate but Stay Resident | Allows programs to stay in memory after termination. |

Interrupt 21h -- Function Request

| 0h | Program Terminate | Terminates the execution of a program |
| --- | --- | --- |
| 1h | Keyboard Input with Echo | Read a character from the keyboard into AL with echo |
| 2h | Display Output | Display the character in DL to the screen |
| 5h | Printer Output | Outputs the character in DL to the printer |
| 8h | Keyboard Input without Echo | Read a character from the keyboard into AL without echo |
| 9h | Print String | Display the string characters addressed by DX to the screen |
| 0Ah | Read String | Read a string from the keyboard into buffer addressed by DX |
| 25h | Set Vector | Sets the address of an interrupt number in the IVT |
| 2Ah | Get Date | Returns the day of the week, year, month and date |
| 2Bh | Set Date | Sets the date |
| 2Ch | Get Time | Returns the time: hours, minutes, seconds, and hundredths |
| 2Dh | Set Time | Sets the time |
| 35h | Get Vector | Obtains the address of an interrupt number from the IVT |
| 39h | Create Subdirectory (MKDIR) | Creates the specified directory. |
| 3Ah | Remove Subdirectory (RMDIR) | Removes the specified directory |
| 3Bh | Change Current Directory (CHDIR) | Changes the current directory to the specified directory. |
| 3Ch | Create a File | Creates a new file |
| 3Dh | Open a File | Opens a file |
| 3Eh | Close a File Handle | Closes the specified file handle |
| 3Fh | Read from a File | Transfers the specified number of bytes from a file to a buffer |
| 40h | Write to a File | Transfers the specified number of bytes from a buffer into a file |
| 41h | Delete a File from a Directory | Removes a directory entry associated with a file name |
| 47h | Get Current Directory | Places the full path name of the current directory in the area pointed by DS:SI. |
| 48h | Allocate Memory | Allocates the requested number of paragraphs of memory |
| 49h | Free Allocated Memory | Frees the specified allocated memory |
| 4Ch | Terminate a Process (EXIT) | Terminates the current process and transfers control to the invoking process. |

Next, we briefly describe some of the DOS INT 21h functions.

|  | Function | Input | Output |
| --- | --- | --- | --- |
| 0BH | Check keyboard buffer | AH = 0BH | AL = 00H - keyboard buffer is empty AL = FFH - keyboard buffer is not empty |
| 0CH | Clear keyboard buffer | AH = 0BH . AL = 01H, 06H, 07H, 08H, or 0AH | Clears keyboard buffer and performs appropriate function depending on AL content |
| 05H | Print a character | AH = 05H DL = ASCII code of the character | None |
| 2AH | Get Date | AH = 2AH | AL = Day of the week (0=SUN, 6=SAT) CX = Year (1980-2099) |
| 2BH | Set Date | AH = 2BH; CX = Year (1980-2099). DH = Month; DL = Day (1-31). | AL = 00h, if the date is valid FFh, if the date is not valid |
| 2CH | Get Time | AH = 2CH | CH = Hours (0-23); CL = Minutes. DH = Seconds; DL = Hundredths. |
| 2DH | Set Time | AH = 2DH. CH = Hours (0-23); CL = Minutes. DH = Seconds; DL = Hundredths. | AL = 00h, if the time is valid AL = FFh, if the time is not valid |

Extended Keyboard Keys

The IBM PC keyboard has several keys that are not the ASCII characters. These keys include the function keys, cursor arrows, Home, End, etc. These keys are called extended keys. When an extended key is pressed, the first byte placed in the keyboard buffer is 00H and the second byte is the keyboard scan code for the key. To read a character from the keyboard using DOS functions, extended keys require two function calls, as shown in the following procedure:

Read the next character code into AL using INT 21h, function 08h if (AL<>0) then AL = ASCII code (ASCII character) else read the scan code of the extended key into AL using INT 21h, function 07h AL = scan code (extended key character) end if

A Time Display Program

As an example of using interrupt routines, we write a program that displays the current time. When the computer is powered up, the current time can be entered by the user or supplied by a real-time clock circuit that is battery powered. This time value is kept in memory and updated by a timer circuit using interrupt 8. A program can call the DOS INT 21h, function 2Ch, to access the time. Our time display program has the following steps:

- Obtain the current time

- Convert the hours, minutes, and seconds into ASCII digits

- Display the ASCII digits

The procedure GET_TIME is used to get the current time and the procedure CONVERT is used to convert a number into ASCII. It first divides the number in AL by 10. This puts the ten's digit value in AL and the unit's digit value in AH (note that the input value is less than 60). Then, it converts both digits to ASCII. These two procedures are shown below:

Example: Procedures GET_TIME and CONVERT.

PUBLIC GET_TIME .MODEL SMALL .CODE GET_TIME PROC MOV AH, 2CH ; CH=hour, CL=min, DH=sec INT 21H ; convert hours into ASCII and store MOV AL, CH CALL CONVERT MOV [BX], AX ; convert minutes into ASCII and store MOV AL, CL CALL CONVERT MOV [BX+3], AX ; convert seconds into ASCII and store MOV AL, DH CALL CONVERT MOV [BX+6], AX RET GET_TIME ENDP CONVERT PROC XOR AH, AH MOV DL, 10 DIV DL ; AH has remainder, AL has quotient OR AX, 3030H ; convert to ASCII RET CONVERT ENDP END

To display the current time, a time buffer, TIME_BUF, is initialized with 00:00:00. The procedure GET_TIME is then called to store the current time in the time buffer. Then, the time_buffer is displayed using INT 21h, function 9. This program is shown below:

Example: Display current time.

EXTRN GET_TIME: NEAR .MODEL SMALL .STACK 100H .DATA TIME_BUF DB '00:00:00$'; .CODE MAIN PROC MOV AX, @DATA ; initialize DS MOV DS, AX ; get and display time LEA BX, TIME_BUF ; BX points to TIME_BUF CALL GET_TIME ; put current time in TIME_BUF LEA DX, TIME_BUF ; display time MOV AH, 9 INT 21H MOV AH, 4CH ; return to dos INT 21H MAIN ENDP END MAIN

User Interrupt Procedures

To make the time display program more interesting, let us write a second version that displays the time and updates it every second. One way to continuously update the time is to execute a loop that keeps obtaining the time via INT 21h, function 2Ch and displaying it. The problem here is to find a way to stop the program.

Instead, this can be done by writing a routine for interrupt 1Ch. This interrupt is generated by the INT 8 routine which is activated by a timer circuit about 18.2 times a second. When our interrupt routine is called, it will get the time and display it.

Set Interrupt Vector

To set up an interrupt routine we need to do the following steps:

- Save the current interrupt vector

- Place the vector of the user procedure in the interrupt vector table

- Restore the previous vector before termination the program

We use the INT 21h, function 35h, to get the old vector and function 25h to set up the new interrupt vector.

Set interrupt Vector: INT 21h, function 25h ; store interrupt vector into vector table Input: AH = 25h AL = interrupt number DS:DX = interrupt vector Output: none

Get interrupt Vector: INT 21h, function 35h ; obtain interrupt vector from vector table Input: AH = 35h AL = interrupt number Output: ES:BX = interrupt vector

The following procedure, SETUP_INT, saves an old interrupt vector and sets up a new vector. It gets the interrupt number in AL, a buffer to save the old vector at DS:DI, and a buffer containing the new interrupt vector at DS:SI. By reversing the two buffers, SETUP_INT can also be used to restore the old vector.

Example: Procedure SETUP_INT for setting interrupt vector.

PUBLIC SETUP_INT .MODEL SMALL .CODE SETUP_INT PROC ;saves old interrupt vector and sets a new one ;input: AL = interrupt number ; DI = address of buffer for old vector ; SI = address of buffer containing new vector ; save old interrupt vector MOV AH, 35H ;get vector INT 21H MOV [DI], BX ;save offset MOV [DI+2], ES ;save segment ;setup new vector MOV DX, [SI] PUSH DS MOV DS, [SI+2] MOV AH, 25H ;set vector INT 21H POP DS ;restore DS RET SETUP_INT ENDP END

Cursor Control

Each display of the current time by INT 21h, function 9, will advance the cursor. If a new time is then displayed, it appears at a different screen position. So, to view the time updated at the same screen position we must restore the cursor to its original position before we display the time. This is achieved by determining the current cursor position; then, after each print string operation, we move the cursor back. We use INT 10h, functions 3 and 2, to save the original cursor position and to move the cursor to its original position after each print string operation.

Move Cursor: INT 10h, function 2 Input: AH = 2 BH = page number DH = row number DL = column number Output: none

Get Cursor Position: INT 10h, function 3 Input: AH = 2 BH = page number Output: DH = row number DL = column number CH = starting scan line for cursor CL = ending scan line for cursor

Interrupt Procedure

The interrupt procedure, TINE_INT, is written to perform the following steps:

- Set DS

- Get new time

- Display time

- Restore cursor position

- Restore DS

The main procedure is written to have the following steps:

- Save the current cursor position

- Set up the interrupt vector for the interrupt procedure, TIME_INT

- Wait for a key input

- Restore the old interrupt vector and terminate

To do step 2, we use OFFSET and SEG to obtain the offset and segment of procedure SETUP_INT; the vector is then stored in the buffer NEW_VEC. The procedure SETUP_INT, is called to set up the vector for interrupt type 1Ch, timer tick. The interrupt 16h, function 0 is used for step 3, key input. Procedure SETUP_INT is again used in step 4; this time SI points to the old vector and DI points to the vector for TIME_INT.

After setting up the cursor and interrupt vectors, the main procedure just waits for a keystroke. In the meantime, the interrupt procedure, TIME_INT, keeps updating the time whenever the timer circuit ticks. After a key is hit, the old interrupt vector is restored and the program terminates. The complete program is given below.

Example: Display updated time program.

EXTRN GET_TIME:NEAR, SETUP_INT: NEAR .MODEL SMALL .STACK 100H .DATA TIME_BUF DB '00:00:00$' CURSOR_POS DW ? NEW_VEC DW ?,? OLD_VEC DW ?,? .CODE MAIN PROC MOV AX, @DATA MOV DS, AX ;save cursor position MOV AH, 3 MOV BH, 0 INT 10H MOV CURSOR_POS, DX ; setup interrupt procedure MOV NEW_VEC, OFFSET TIME_INT MOV NEW_VEC+2, SEG TIME_INT LEA DI, OLD_VEC LEA SI, NEW_VEC MOV AL, 1CH CALL SETUP_INT ;read keyboard MOV AH, 0 INT 16H ;restore old interrupt vector LEA DI, NEW_VEC LEA SI, OLD_VEC MOV AL, 1CH CALL SETUP_INT ;exit to DOS MOV AH, 4CH INT 21H MAIN ENDP ; TIME_INT PROC PUSH DS MOV AX, @DATA MOV DS, AX ; get new time LEA BX, TIME_BUF CALL GET_TIME ; display time LEA DX, TIME_BUF MOV AH, 9 INT 21H ;restore cursor position MOV AH, 2 MOV BH, 0 MOV DX, CURSOR_POS INT 10H POP DS IRET TIME_INT ENDP END MAIN END

File Processing

In this section, we will discuss a group of INT 21h functions called file handle functions. We will describe what is meant by a file handle, file errors, opening and closing a file, reading from and writing to a file.

File Handle

When a file is opened or created in a program, DOS assigns it a unique number called the file handle. This number is used to identify the file, so the program must save it. There are five predefined file handles as shown in the following table:

| File Handle | Device |
| --- | --- |
| 0 | keyboard |
| 1 | screen |
| 2 | error output -- screen |
| 3 | auxiliary device |
| 4 | printer |

File Errors

There are many opportunities for errors in INT 21h file handling. DOS identifies each error by a code number. In the functions we describe here, if an error occurs then the CF is set and the code number appears in AX. The following table contains the most common file-handling errors:

| Hex Error Code | Meaning |
| --- | --- |
| 1 | invalid function number |
| 2 | file not found |
| 3 | path not found |
| 4 | all available handles in use |
| 5 | access denied |
| 6 | invalid file handle |
| C | invalid access code |
| F | invalid drive specified |
| 10 | attempt to remove current directory |
| 11 | not the same device |
| 12 | no more files to be found |

Opening a New File

Before a file can be used, it must be opened. To create a new file or rewrite an existing file, the user provides a filename and an attribute and DOS returns a file handle.

Open a New File/Rewrite a File: INT 21h, function 3Ch Input: AH = 3Ch DS:DX = address of file name, which is an ASCII string (a string ending with a 0 byte) CL = attribute Output: if successful, AX = file handle Error if CF = 1, error code in AX (3, 4, or 5)

The file name may include a path; for example C:\COE205\EX.ASM. Possible errors for this function are 3 (path does not exist), 4 (all file handles in use), or 5 (access denied, which means either that the directory is full or the file is read-only file). A file attribute is a byte that specifies some attributes for files as shown below:

| Bit | Meaning if Set |
| --- | --- |
| 0 | Read-only file |
| 1 | Hidden file |
| 2 | DOS system file |
| 3 | Volume label |
| 4 | Subdirectory |
| 5 | Archive bit |
| 6 | Not used |
| 7 | Not used |

Example: Write a program to open a new read-only file called FILE1.

.model small .stack 100h .data FNAME DB 'FILE1', 0 HANDLE DB ? .code .startup MOV AH, 3Ch ; open file function LEA DX, FNAME ; DX has filename address MOV CL, 1 ; read-only attribute INT 21H ; open file MOV HANDLE, AX ; save handle or error code JC OPEN_ERROR ; jump if error ... .exit END

Opening an Existing File

To open an existing file, there is another function:

Open an Existing File: INT 21h, function 3Dh Input: AH = 3Dh DS:DX = address of file name, which is an ASCII string (a string ending with a 0 byte) AL = access code: 0 means open for reading 1 means open for writing 2 means open for both Output: if successful, AX = file handle Error if CF = 1, error code in AX (2, 4, 5 or 12)

Closing a File

After a file is processed, it should be closed. This frees the file handle for use with another file.

Close a File: INT 21h, function 3Eh Input: BX = file handle Output: if CF = 1, error code in AX (6)

Example: Write some code to close a file whose handle is stored in variable HANDLE.

MOV AX, 3Eh ; close file function MOV BX, HANDLE ; get handle INT 21H ; close file JC CLOSE_ERROR ; jump if error ...

Reading from a File

The following function reads a specified number of bytes from a file and stores them in memory.

Read from a File: INT 21h, function 3Fh

Input:	AH = 3Fh

BX = file handle

CX = number of bytes to read

DS:DX = memory buffer address

Output:	AX = number of bytes actually read

if AX=0 or AX < CX, end of file encountered

Error if CF = 1, error code in AX (5, 6)

Example: Write some code to read a 512-byte from a file. Assume file handle is stored in variable HANDLE, and BUFFER is a 512 byte buffer.

.data

HANDLE DW	?

BUFFER DB 512 DUP(0)

...

.code

...

MOV AX, 3Fh	; read file function

MOV BX, HANDLE	; get handle

MOV CX, 512	; read 512 bytes

INT 21H		; read file, AX = bytes read

JC READ_ERROR	; jump if error

...

Writing to a File

Function 40h writes a specified number of bytes to a file.

Write to a File: INT 21h, function 40h Input: AH = 40h BX = file handle CX = number of bytes to write DS:DX = data address Output: AX = number of bytes written if AX=0 or AX < CX, error (full disk) Error if CF = 1, error code in AX (5, 6)

Function 40h writes data to a file, but it can also be used to send data to the screen or printer (handles 1 and 4 respectively).

Example: Use function 40h to display a message on the screen.

.data MSG DB 'Display This Message' ... .code ... MOV AX, 40h ; write file function MOV BX, 1 ; screen file handle MOV CX, 20 ; length of message LEA DX, MSG ; get address of MSG INT 21H ; display MSG ...